Sierra-Sacramento

Version – January 26, 2026

Airspace Deconfliction Guidelines

Contents

Purpose Statement

Guideline – Airspace Deconfliction
    Communication Protocols
     Airspace Management Guidelines for Incident Traffic Areas (ITA’s)

Appendix
     Term Definition (Detailed)
     Glossary (Abbreviated)
     Guide Cards & Scripts for ITA Operations
     Incident Traffic Area (ITA) Diagram
     Incident Command System (ICS) Organization Chart
     UAS Identification (Naming Convention for Large/Complex Incidents)

PURPOSE

This guideline is intended to ensure the safe and effective deployment of one or multiple UAS in an airspace shared with crewed-aircraft or other UAS.  This guideline outlines airspace definitions, communication protocols, and response guidelines designed to ensure safety for UAS and crewed-aircraft operators, pilots, and ground personnel.

GUIDELINE

Section 1 - Communication Protocols

Effective communication is the single most important aspect of airspace deconfliction.  Communication protocols are intended to ensure direct pilot to pilot interactions, and to communicate such control mechanisms as fences, ceilings, floors, routes, and ‘hold clear’ areas to secure vertical and horizontal separation between aircraft.

  - Two-way communications between all aircraft are required in order for UAS to operate in air space shared with crewed-aircraft. If a crewed aircraft (helicopter) arrives in the airspace, UAS pilots will attempt to make radio contact. If UAS pilots are unable to contact crewed-aircraft, they will land their aircraft until positive contact has been established.

    - All aircraft working within the airspace will be in contact with the Airspace Coordinator (“AirCo”) and with the other aircraft working in the airspace. This can be via AM or FM radio frequencies.

  - All crewed-aircraft responding to an incident where UAS are or may be operating shall establish communications with the Incident Commander (IC), or designee (eg. Air Operations, AirCo, UAS pilot, etc.) prior to entering the Incident Traffic Area (ITA) to establish whether UAS aircraft are in operation over the incident.

  - UAS pilots shall contact the Incident Commander (IC), Airspace Coordinator (“AirCo”), or designee, prior to taking off while in the Incident Traffic Area.

    - The IC (or designee) shall notify the controlling ECC or Dispatch Center of active UAS operations and what channel is being utilized for air-to-air communications.

    - The ECC or Dispatch Center shall notify all incoming aircraft assigned to the incident of active UAS operations and the assigned air-to-air frequency.

    - UAS pilots shall also establish communication with crewed-aircraft directly or via the Airspace Coordinator prior to launching.

    - In addition to monitoring normal incident frequencies (Command, Tac, etc.) UAS and crewed-aircraft pilots should also monitor CALCORD as able. This will provide a hailing channel between UAS and crewed-aircraft pilots.  UAS pilots should also monitor CALCORD during training.

    - Additionally, if no IC is present, or if unable to establish contact with IC, crewed-aircraft should approach incidents where an Incident Traffic Area (ITA) exists, or may exist, in a “UAS-Friendly Approach” until it is established that no UAS are operating at scene. (See definition in Appendix – “Term Definitions #11”).

  - UAS aircraft will identify themselves by their Agency followed by their Unit identifier. Example – “Placer County UAS-1” or “CAL FIRE NEU-401”.

    - For large incidents, or incidents with multiple UAS, Incident Command (IC) or the Airspace Coordinator may elect to utilize the Incident Command System (ICS) nomenclature for all UAS (ie. UR-41; UF-11, etc.).  (Reference the ICS Field Operations Guide (FOG) Manual for additional information.)

  - As new aircraft arrive or launch within the ITA, a radio or face-to-face briefing shall be conducted between crewed aircraft pilots, uncrewed aircraft pilots, and/or the Airspace Coordinator that includes the following information:
 	
Incident Traffic Area (ITA) Briefing

I – Identifier (call-sign) of all aircraft currently operating

H – Hazards in the area

M – Mission Type (ie. Recon, Rescue, External load, Mapping, etc.)

A – Altitude(s) – Vertical Separation - (What aircraft are operating at what altitude)

B – Boundaries for operation – Horizontal Separation (Fences, etc.)

R – Radio frequency for air-to-air communications

I – Information/intelligence to pass along

C – Coordinator establishment/acknowledgement – Airspace Coordinator assuming duties of aerial supervision and coordination.

This briefing should occur upon arrival of a second aircraft at an incident and with every additional arriving aircraft so that all aircraft operating within the Incident Traffic Area will be adequately briefed.

  - All aircraft operating within the Incident Traffic Area (ITA) will maintain communications via the agreed upon air-air frequency until they land or depart the Incident Traffic Area.  

Notification should be made when an aircraft departs the Incident Traffic Area or completes their mission and lands for more than a few minutes so other pilots understand their aircraft has departed the ITA.

    - UAS pilots do not need to make notifications if landing for a battery change or other short-duration landing, unless requested to do so by crewed aircraft, the Airspace Coordinator, or if landing will impact safety and/or situational awareness.

Section 2 - Airspace Management Guidelines for Incident Traffic Areas (ITA’s)

The integration of crewed and uncrewed aircraft significantly increases operational complexity and safety risk. Robust coordination among all aerial responders is required to prevent collisions, mission conflict, and communication failures. As incidents grow in size and complexity, disciplined airspace coordination is essential to safeguard civilians, first responders, and flight crews.

The following guidelines are designed to ensure a coordinated and controlled airspace over incidents with mixed aircraft or incidents with multiple UAS aircraft.

  - Airspace with mixed aircraft (ie. crewed & uncrewed) or with multiple UAS aircraft should designate and establish an Airspace Coordinator (“AirCo”). The Airspace Coordinator will assume coordination of air resources and ensure that aircraft operating over the incident have clearly identified working areas. 

The Airspace Coordinator is similar in function and in ICS organization to an Air Tactical Group Supervisor (ATGS “Air Attack”) or an “Air Boss” in Law Enforcement.

    - This role can be assumed by a crewed aircraft, a UAS pilot, or a non-aviation personnel.

  - The Airspace Coordinator will designate defined work areas for various aircraft utilizing horizontal and/or vertical separation elements 

Work zones and airspace assignments may change throughout the incident based on various incident parameters such as victim location, incident needs (ie. recon, rescue, personnel delivery, etc.), aircraft capability, and weather/visibility considerations.

    - Crewed and uncrewed (UAS) aircraft should be separated by a minimum of 500 feet vertically and/or horizontally.

      - This separation can be decreased in special circumstances if agreed upon by all pilots/operators.

    - Horizontal separation work zones should be delineated by easily recognizable and identifiable features that can serve as “virtual fences” such as roads, towers, buildings, topographical features, and etc. 

Example –  AirCo: “UAS will stay north of Inspiration Point, and helicopters will stay south.”

    - Aircraft will not leave their assigned area (ie. cross a fence or altitude separation) unless specifically authorized by the Airspace Coordinator, and notification has been made to other potentially impacted aircraft.

Example of UAS Pilot requesting authorization to deviate from their assigned flight area:

| Unit | Function/Purpose of traffic | Traffic |
| --- | --- | --- |
| CHP H-24 | Helicopter requesting authorization from AirCo to fly PAST an assigned fence | “Confluence AirCo, CHP H-24. I am requesting clearance to fly north of River Fence direct to the victim’s location for a hoist operation” |
| AirCo | AirCo is ensuring other aircraft in airspace are aware of the requested deviation and are clear. | “Placer County UAS-1, be advised that CHP H-24 will be transiting north of River Fence, direct to the victim location. Confirming you are clear of that area?” |
| UAS Pilot 1 | Participating aircraft confirms the change to airspace and lands UAS. | “UAS 1 copies. We are landing right now and are clear of that area. We copy that CHP H-24 will be north of River Fence in the area of the victim’s location and we have them in sight.” |
| AirCo | AirCo authorizes helicopter to operate in the new area. | “CHP H-24, you are cleared north of River Fence direct to the victim location. UAS-1 has landed and has you in sight.” |
| CHP H-24 | CHP H-24 confirms authorization for requested deviation. | “CHP H-24 copies. Enroute to victim location” |

  - Airspace Coordinators should utilize tools to increase their Situational Awareness, including: 
• Software programs that utilize ADS-B to show real-time locations of crewed aircraft (ie.  
     Foreflight or FlightRadar 24, etc.)
• Other software management programs (such as DroneSense) that show real-time 
     locations of UAS aircraft.

  - UAS aircraft should utilize beacons on their aircraft when operating in airspace shared with crewed aircraft to increase their visibility.

  - All aircraft, whether crewed or uncrewed, are to fly Visual Flight Rules (VFR) ‘See & Avoid’.

    - Per Federal Aviation Administration (FAA) FAR Part 107, the legal imperative for uncrewed aircraft is to ‘see and to avoid’ crewed aircraft.

  - The standard is for crewed aircraft to fly above uncrewed aircraft unless special circumstances exist and it is agreed upon by all pilots/operators.

  - All aircraft must utilize the same ground reference (AGL or MSL).

    - Incident Traffic Areas (ITA’s) will generally utilize altitude expressed Above Ground Level (AGL).

    - There may be times with it is more effective to utilize altitude and vertical separation in terms of Mean Sea Level (MSL).  This includes environments with large altitude differences (ie. over a large canyon) or environments largely staffed by crewed aircraft (ie. Large SAR operations, etc.).

    - UAS operators must be able to calculate their altitudes MSL in order to operate in a MSL environment.

  - Only aircraft operated by participating emergency or public response should be operated within an ITA, under the authorization of the Incident Commander.

    - Legal Reference -  Ref. PC 402(a)(2): California Code, PEN 402.

APPENDIX

Term Definitions (Detailed)

Airspace or ITA-specific definitions

For the purposes of this guideline, the following terms shall be defined as:

- Airspace Coordinator (“AirCo”)

The Airspace Coordinator is the controlling entity responsible for ensuring all pilots know and understand the parameters and separation zones over a given an incident. This position acts as aerial supervisor (or “air traffic control” / “Air Boss”) to establish and ensure safe separation and sufficient communication between aircraft, whether crewed (helicopters) or uncrewed (UAS).  This position may be a specific-assigned position (ie. standalone) or an additional function or role assigned to a UAS pilot or crewed aircraft.

  - For most incidents, an Airspace Coordinator should be assigned to an experienced UAS pilot who, ideally, is positioned at or near the Incident Command Post (ICP) and can safely manage the airspace in addition to their other roles (ie. piloting a UAS).  The role of Airspace Coordinator should take precedence over all other roles, unless special circumstances exist.

  - For larger or more complex incidents or incidents with multiple UAS or crewed aircraft, the role of Airspace Coordinator should be assigned to an individual with no other roles or positions.

  - The Airspace Coordinator can optionally be assigned to crewed aircraft, such as a rescue or law enforcement helicopter.  However, due to the nature and intensity of helicopter and rescue work, the role of Airspace Coordinator is best assumed by an individual on the ground who is stationary and who is informed on the ground-based actions and actions of the Incident Command Post (ICP).

- Blind Calls

 A Blind Call is a one-way radio transmissions directed to alert aircraft that may be operating in a shared area to announce arrival direction/location, altitude, and intention. Blind calls are made by aircraft (helicopters) arriving over a new incident or by UAS pilots to announce their intentions to operate in an area if no contact has been made with aircraft and no aircraft are known to be operating in the airspace.  The purpose of a blind call is to notify any aircraft that may be in the area that a new aircraft is entering the airspace.

If other aircraft are known to be operating in the airspace, then direct communication must be established prior to launching (if UAS inside the airspace) or prior to entering the airspace (if a crewed aircraft).

- Ceiling or Floor

An assigned altitude (high or low) not to cross. Utilized for vertical separation of aircraft. Can be referenced as Above Ground Level (AGL) or as above Mean Sea Level (MSL). (e.g. “UAS shall maintain a ceiling of 400 feet AGL or below”).

  - If Mean Sea Level (MSL) is utilized, then a standard altimeter reading must be referenced.

  - Above Ground Level (AGL) is generally preferred in UAS operations.  However, there may times, such as when multiple crewed aircraft are operating within the same airspace, that Mean Sea Level (MSL) will be utilized. Altitude limits are generally identified in 500-foot increments (e.g. 4000; 4500; 5000, etc.)

  - The altitude reference utilized for operations within an Incident Traffic Area (ITA) shall be agreed upon by all pilots prior to multi-aircraft operations. 

Example [MSL]: “Helicopter H-24, River Incident Airspace Coordinator on CALCORD”
“River Airspace Coordinator, H-24, go ahead”
“H-24, River Airspace Coordinator, current altimeter is 2997. You are cleared in to the River ITA at two thousand, five hundred feet or above. UAS Unmanned Romeo 41 is operating two thousand and below.”
“River Airspace Coordinator, H-24 copies altimeter 2997 and cleared in at two thousand, five hundred or above, and Unmanned Romeo 41 is two thousand and below.”

- Corridor 

A passage through another air space. May be defined by lateral description as well as vertical references. Used to allow aircraft to pass through a restricted airspace, typically to reach a specific landing zone (LRZ) or similar specific need.  (e.g. “State Parks UAS 3 may transit through helicopter airspace if over Highway 20 at an altitude of 100 feet or below”).

- Fence

A line or geographical reference separating a zone or an area to encompass specific flight operations or to exclude specific operations. Utilized to create or manage horizontal separation. Fences are not to be crossed unless specific permissions are granted. (e.g. “Falcon 30, River Airspace Coordinator, we will be utilizing the centerline of the river as a fence. Helicopters will stay North of the river and UAS will stay South of the river”)

- Fire Traffic Area (FTA) 

A Fire Traffic Area (FTA) is a communication protocol utilized on wildfires to provide a safe operating environment for aircraft engaged in firefighting activities. It is a temporary designation set up to manage air traffic in the vicinity of active fires.

  - This guideline will not address UAS or flight operations within an FTA.  For additional information about Fire Traffic Areas, see NWCG Standards for Aerial Supervision – PMS 505.

- Hold out area 

An area temporarily restricted to flying.  (e.g. “All UAS need to hold out of Zone 2 until the hoisting operation is complete”).

- Incident Traffic Area (ITA) 

An Incident Traffic Area (ITA) is similar in purpose and function to a Fire Traffic Area (FTA – Utilized on wildland fire incidents) or a military “Battlespace” but is designed for non-wildfire incidents (e.g. rescues, flooding, earthquake, etc.) and may or may not have a controlling aircraft acting as an aerial supervisor.

  - An Incident Traffic Area is intended to assist in the organization and deconfliction between crewed and uncrewed aircraft when operating in the same airspace, or when multiple aircraft are flying in the same airspace.

  - Incident Traffic Areas (ITA’s) will:

    - Be established by an Incident Commander (IC), pilot, or UAS operator over an incident where UAS are operating, or may operate, particularly if crewed and uncrewed aircraft are operating over the same incident.

    - Be defined in terms of Above Ground Level (AGL) or Mean Sea Level (MSL).  AGL or MSL must be agreed upon and communicated by all pilots prior to multiple aircraft operating in an Incident Traffic Area (ITA).  Simple or non-complex Incident Traffic Areas will generally be referenced in AGL.

    - Incorporate an area of no less than 1 mile radius from the incident (if small/localized) or 1 mile from outer boundaries of where UAS operations are planned or expected to occur.

    - When appropriate, maps should be created and disseminated to participating agencies and nearby response agencies defining an incident’s Incident Traffic Area.

    - These guidelines are intended for ALL UAS operating within the ITA, including media, public utility, and other emergency response UAS aircraft.

- Route 

A designated or pre-established pathway between two or more points for aircraft to follow.  May be singular or bidirectional. Will commonly include an assigned altitude.  

Example – The “River Route” will be utilized for UAS to transit from the Landing Zone (LZ) to the assigned work area. All Northbound traffic on the River Route will fly below 100 feet AGL or below, and Southbound traffic will fly 200 feet AGL or above.”

- Temporary Flight Restriction (TFR) 

A Temporary Flight Restriction (TFR) is a restriction placed on airspace by the Federal Aviation Administration (FAA) to prohibit or limit certain flight operations in a specific area for a temporary period of time. TFRs are typically issued for:

- Protecting VIPs (e.g., presidential or high-level government officials' movements)

- Special events (e.g., airshows, sporting events, or public gatherings)

- Security concerns (e.g., restricted areas following security threats)

  - A TFR can either restrict all flight traffic in the area or allow certain types of flights with specific permissions. The TFR will specify the geographic boundaries, altitude limits, and the time frame during which the restriction is in effect. Pilots must be aware of and comply with TFRs to avoid penalties, including fines or other legal consequences.

  - Temporary Flight Restrictions are usually communicated through notices such as NOTAMs (Notices to Air Mission), and pilots can check current TFRs through FAA resources or aviation apps.

  - TFR’s have specific legal definitions and requirements for aerial operators and pilots.

  - This guideline will not address UAS or flight operations within a TFR.

- “UAS-Friendly Approach” 

Steps taken by crewed aircraft to increase their safety as they approach a new incident where public safety or civilian UAS may be present.

  - A UAS-Friendly Approach includes:

    - Establish contact with IC or ECC/Dispatch prior to entering a 1-mile ring around the incident to confirm whether UAS are operating over the incident.

    - If no contact with ground personnel has been made, make a Blind Call on the Incident Command channel, Tactical channel, and/or CALCORD to notify on-scene personnel, including any UAS operators, of your arrival.

    - Approach the incident >500 feet AGL

    - Slow your approach as you enter the airspace over the incident and make at least one orbit at a “high” altitude (500-1000’ AGL) to allow any UAS operators (public safety or civilian) to see the incoming aircraft and land their UAS (see and avoid).

    - Situational awareness - Making a conscientious scan of the area for UAS as you arrive.

Glossary (Abbreviated)

| ADS-B | Automatic Dependent Surveillance–Broadcast - An advanced aviation surveillance technology that operates by enabling aircraft to determine their position via satellite navigation and periodically broadcast this information, along with other pertinent data, to ground stations and other aircraft. |
| --- | --- |
| AGL | Above Ground Level - Refers to the altitude or height of an object, such as an aircraft or drone, measured relative to the underlying ground surface directly beneath it. Distinct from MSL – Mean Sea Level. |
| Air Boss | See ATGS |
| Airspace Coordinator (“AirCo”) | Airspace Coordinator - Controlling entity responsible for ensuring all pilots know and understand the parameters and separation zones over an Incident Traffic Area (ITA). Acts as the Incident Air Traffic Control or ‘Air Boss’. |
| AOBD | Air Operation Brach Director - Responsible for the coordination, safety, and efficient use of air resources, ensuring that aviation support contributes effectively to the overall success of the incident response. The AOBD works in base camp and supervises the ATGS and Helicopter Supervisors. |
| ATGS | Air Tactical Group Supervisor - The ATGS is responsible for managing and coordinating the use of aircraft during a wildfire or other emergency incident. Their primary duties include: Air operations coordination, safety, communication, and resource management. Occasionally referred to as an ‘Air Boss’. |
| Blind Calls | One-way radio transmissions directed towards aircraft that may be operating in a shared area to announce your arrival direction/location, altitude, and intention. |
| CALCORD | California On-Scene Emergency Coordination – (156.075) A common tactical channel or frequency primarily utilized for interagency communication during emergencies. |
| Ceiling or Floor | Assigned altitudes (high or low) not to cross. |
| Command channel or frequency | The primary channel for communicating between the incident and responding resources and the ECC or Dispatch center. Distinct from Tactical channel. |
| Corridor | A passage through another air space. May be defined by lateral description as well as vertical references. |
| Crewed Resource or Aircraft | An aircraft with a pilot, crew, or personnel on board. Distinct from Uncrewed Resource (UAS, etc.) |
| ECC | Emergency Command Center - A dispatch center staffed with Captains or above. |
| Fence | A line or geographical reference separating a zone or an area to encompass specific flight operations or to exclude specific operations. Utilized to create or manage horizontal separation. |
| Fire Traffic Area (FTA) | Fire Traffic Area - is a communication protocol utilized near wildfires to provide a safe operating environment for aircraft engaged in firefighting activities. |
| FOG | Field Operations Guide - a comprehensive manual designed to assist emergency response personnel in effectively applying the Incident Command System (ICS) during various incidents and planned events. |
| Hold out area | An area temporarily restricted to flying. |
| Horizontal separation | An aerial supervision concept that refers to the lateral distance maintained between aircraft to prevent collisions. Distinct from Vertical separation. |
| Incident Traffic Area (ITA) | An Incident Traffic Area (ITA) is similar in purpose and function to an FTA or a military “Battlespace” but is designed for non-wildfire incidents (e.g. rescues, flooding, earthquake, etc.) and may or may not have a controlling aircraft acting as an aerial supervisor. |
| ICP or CP | Incident Command Post or Command Post - The location from which the incident commander (IC) and their team coordinate all response activities during an emergency. The ICP serves as the central hub for communication, planning, resource management, and logistics. |
| ICS | Incident Command System (ICS) - A standardized framework for managing emergencies and planned events, enabling responders from various agencies to collaborate effectively under a unified organizational structure. |
| Landing Zone (LZ) | Landing area designated for helicopters and/or UAS aircraft. |
| LEO | Law enforcement officer |
| MSL | Mean Sea Level - The average height of the ocean's surface serving as a standard reference point for measuring elevations and altitudes. In aviation, MSL is used as a datum for altitude measurements, ensuring consistency in navigation and flight operations. |
| NOTAM | Notice to Airmen - A notice issued by FAA to inform pilots and flight personnel about essential information concerning the establishment, condition, or change in any aeronautical facility, service, procedure, or hazard. NOTAMs are used to alert airspace users to hazards along their route, both in the air and on the ground. |
| Remote ID | A "digital license plate" for drones, enabling them to broadcast identification and location information in real-time during flight. |
| Route | A designated or pre-established pathway between two or more points for aircraft to follow. |
| Tactical channel or frequency | A secondary radio channel assigned to a specific incident for resources to utilize for incident-related channel. Distinct from a Command channel or frequency. |
| Temporary Flight Restriction (TFR) | A Temporary Flight Restriction (TFR) is a restriction placed on airspace by the Federal Aviation Administration (FAA) to prohibit or limit certain flight operations in a specific area for a temporary period of time. |
| UAS | Unmanned Aerial Systems - aka Drone(s) |
| “UAS-Friendly Approach” | Steps taken by crewed aircraft to increase their safety as they approach a new incident where public safety or civilian UAS may be present. |
| Uncrewed resource or aircraft | UAS, drones, or other remotely-controlled aircraft with no personnel on board |
| Vertical separation | An aerial supervision concept that refers to the vertical distance maintained between aircraft to prevent collisions. Distinct from Horizontal separation. |
| VHF-AM Frequency | 118 to 136.975 MHz allocated for voice communications for aviation. Distinct from VHF-FM frequencies. |
| VHF-FM Frequency | 150 and 174 MHz – Used by many Public Safety organizations for voice communications. Distinct from VHF-AM frequencies. |
| Visual Flight Rules (VFR) | A set of regulations under which a pilot operates an aircraft in weather conditions generally clear enough to allow the pilot to see where the aircraft is going. Distinct from Instrument Flight Rules (IFR). |

ITA Briefing

Incident Traffic Area (ITA) Briefing

I – Identifier (call-sign) of all aircraft currently operating

H – Hazards in the area

M – Mission Type (ie. Recon, Rescue, External load, Mapping, etc.)

A – Altitude(s) – Vertical Separation - (What aircraft are operating at what altitude)

B – Boundaries for operation – Horizontal Separation (Fences, etc.)

R – Radio frequency for air-to-air communications

I – Information/intelligence to pass along

C – Coordinator establishment/acknowledgement – Airspace Coordinator assuming duties of aerial supervision and coordination.

ICS Organization Chart (UAS Operations)