Sierra-Sacramento

Version – February 9, 2026

UAS Mutual Aid Request Guidelines

Contents

Introduction

Guideline – Interagency UAS Utilization

	Section 1 – Notification and Activation Process

Section 2 – Agency Expectations (Requesting Agency)

Section 3 – Agency Expectations (Sending Agency)

Section 4 – Universal UAS Pilot Classification System

Section 5 – UAS Flying Environment Classification

Section 6 – Standard Mission Types

PURPOSE

This guideline establishes standardized procedures for requesting, deploying, and managing Unmanned Aircraft Systems (UAS) resources under the ROTr Mutual Aid framework.

It further establishes a Universal UAS Pilot Classification System and a UAS Flying Environment Classification (FEC) model to ensure that agencies requesting mutual aid receive appropriately qualified personnel and equipment for the mission environment.

GUIDELINE

Introduction

Complex, long-duration, or technically demanding incidents may exceed the UAS capabilities of a single agency. The purpose of this document is to provide clear notification procedures and standardized classification terminology so that Requesting Agencies may accurately describe operational needs and Sending Agencies may provide properly matched UAS resources.

This guideline aligns with FAA Part 107 regulations, COA operations, ICS doctrine, and best practices outlined in national UAS integration frameworks such as FAA Advisory Circular 107-2A and DHS First Responder UAS operational guidance.

All mutual aid UAS operations shall operate under the operational control of the Incident Commander or their designee. The assigned UAS pilot retains final authority regarding flight safety decisions.

Section 1 – Notification and Request Process

When an agency requires mutual aid UAS assistance, the following process shall be followed:

- A direct phone call shall be made to the Point of Contact (POC) listed on the current ROTr Mutual Aid Roster and/or a request may be made utilizing the ROTr What’s App Notification Group.

- The requesting agency shall provide:

  - Incident name and location

  - Mission type (Fire, Search, Tactical, etc.)

  - Requested Minimum Pilot Level (1–3)

  - Expected Flying Environment Level (1–3)

  - Estimated operational period (if known)

- Following the initial phone request:

  - Dispatch/ECC request should follow prescribed Mutual Aid request procedures (e.g. Request formally made through Dispatch/ECC, Duty Chief, etc.)

  - The Sending Agency shall confirm acceptance, estimated time of arrival (ETA), and resource capability prior to deployment.

Section 2 – Agency Expectations (Requesting Agency)

The Requesting Agency shall:

- Clearly identify mission objectives, deliverables, and special requirements/needs (e.g. IR/Thermal sensors, map production, etc.)

- Identify the desired minimum-required Pilot Level (1-3)

- Identify the expected Flying Environment Level (1-3) classification (if known).

- Ensure airspace deconfliction measures are in place (ITA, Airspace Coordinator, SGI, etc. if applicable).

- Upon arrival of resources, provide a briefing on:

  - Hazards (terrain, wires, towers, obstructions)

  - Aircraft activity (crewed and uncrewed)

  - Weather conditions

  - Operational control structure (ICS chain of command)

  - Airspace Deconfliction plan

Section 3 – Agency Expectations (Sending Agency)

Sending Agencies shall:

- Ensure pilots meet FAA Part 107 or COA requirements.

- Confirm appropriate pilot classification level (1-3).

- Confirm aircraft suitability for requested environment and mission.

- Validate currency, maintenance, and airworthiness of pilot and aircraft.

- Assume responsibility and liability for deployed aircraft and personnel.

  - No personally owned UAS shall be deployed without IC approval.

Section 4 – ROTr UAS Pilot Classification System

This system establishes a tiered-competency model and provides a shared language for pilot competency and capability.

Level 1 Pilot – Advanced / Highly Proficient (“Tier 1” or “Type 1”)

A highly competent, experienced UAS pilot with extensive flight hours. Able to operate competently in complex airspace and multi-aircraft operations. Demonstrates advanced understanding of airspace integration, ICS coordination, night operations, and flying in high-risk environments. Capable of operating in Level 1 environments with no supervision. Competent in various technology operations required for integration and deliverables.

Level 2 Pilot – Intermediate / Operationally Proficient

A competent UAS pilot with moderate operational experience. Capable of operating independently in routine public safety missions and Level 2 environments. May require coordination support in high-complexity environments but demonstrates sound aeronautical decision-making and regulatory knowledge.

Level 3 Pilot – Basic Pilot

A newer or lower-time UAS pilot with foundational training and certification. Best suited for Level 3 environments with low airspace complexity and minimal hazards. May require supervision or operational oversight in dynamic environments.

Section 5 – UAS Flying Environment Classification (FEC)

The following classification describes operational complexity and serves to assist in describing expected mission difficulty.

Level 1 Environment – High Complexity (“Tier 1” or “Type 1”)

Operations in controlled or congested airspace, near airports, within TFRs (with authorization), or in proximity to crewed aircraft operations. May involve BVLOS waivers, nighttime operations, mountainous or canyon terrain, towers/obstructions, reduced visibility, smoke, or adverse weather. Requires advanced airspace coordination and high pilot proficiency.

Level 2 Environment – Moderate Complexity

Operations in Class G airspace with some environmental or operational challenges such as moderate terrain, limited aircraft activity, moderate wind/weather considerations, or limited nighttime operations. Airspace coordination may be required but risk factors are manageable.

Level 3 Environment – Low Complexity

Operations in open Class G airspace with minimal hazards, flat terrain, low population density, daytime VLOS operations, favorable weather, and no nearby airports or aircraft activity. Ideal for training-level or basic mission deployments.

Section 6 – Mission Types

To standardize requests, missions shall be categorized as follows:

Fire (Wildland or Structure)

Includes wildland fire mapping, hotspot identification, structure assessment, firing operations overwatch, and situational awareness. Often conducted in smoke-reduced visibility and at night. Requires IR/Thermal sensor package and IR telemetry.

Search / Missing Person

Grid or targeted search operations in urban, rural, wilderness, or mountainous terrain. May require thermal payloads, extended endurance, or night operations.

Interior Operations (Tactical)

Confined space or structure-based operations, including tactical overwatch or dynamic entry support. May require micro UAS platforms with FPV and highly skilled pilots.

HazMat

Hazardous materials reconnaissance, plume modeling support, and scene assessment without personnel exposure. Often requires stable hover precision and data relay.

Surveillance (Law Enforcement)

Overwatch, perimeter control, suspect tracking, and evidentiary documentation. Requires strict compliance with privacy, legal, and data retention policies.

Other

Any mission not otherwise classified. Requesting agency shall provide clear operational objectives and environment level.